Module 2:

Find the auto correlation of xn=[0, 1, 2, 3]

Solution:  x[n]*x[-n]=[0, 3,8,14,8,3,0]

|  | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 3 | 2 | 1 | 0 |
| 2 | 6 | 4 | 2 | 0 |
| 3 | 9 | 6 | 3 | 0 |

Find cross-correlation of xn=[0, 1, 2, 3], hn=[1, 1, 2, 1]

Solution:  x[n]*h[-n] = [0, 1, 4, 8, 9, 5,3]

|  | 1 | 2 | 1 | 1 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 2 | 1 | 1 |
| 2 | 2 | 4 | 2 | 2 |
| 3 | 3 | 6 | 3 | 3 |

Find whether the following differential equations are representing the equations for LTI systems or not:

- 4d3y(t)dt3+2d2y(t)dt2+3dydt+yt=2dx(t)dt+5x(t)

a3=4, a2=2,a1=3,a0=1 and b1=2, b0=5

The coefficients are constant so TI.

The given system is LTI, if the initial conditions are zero

- Ld2y(t)dt2+Rdy(t)dt+1cyt=x(t)

Here, a2=L,a1=R,a0=1/c and b0=1

The coefficients are constant so TI.

The given system is LTI, if the initial conditions are zero

- 4dydt2+2yt=x(t)

Not matching the format so its not LTI.

- 3d2ydt2+yt=2tx(t)

As one of the coefficients is t which is not constant, its not a LTI system.

Difference Equation: Represent discrete time LTI systems

The standard difference equation is represented by:

aNyn-N+…+a1yn-1+a0yn=bMxn-M+…+b1xn-1+b0x[n]

The generalized form:

k=0Naky[n-k]= k=0mbkx(n-k)

Q1. yn-1+yn=x[n], coefficients are constant and it’s a linear equation. So its LTI.

Q2. yn+yn-1-2yn-2=xn-1+2x[n-2], constant coefficient and linear equation so LTI.

Find the auto correlation function of xt=e-3tu(t)

Solution:

R(τ)=∞∞x1(t)x2(t-τ)dt

Rτ=xτ*x(-τ)

. Evaluate step response of a system

hn=12nu(n)

Solution:

sn=k=-∞nh(k)=k=-∞n12ku(k)=k=0n12k. 1

sn=12n+1-112-1=2-12n     (Formula: k=0nan=an+1-1a-1=1-an+11-a)

Convolution

Q. Find convolution of two signals.

xt=t2+2t+1

yt=t2+3t+4

zt=xt*y(t) (Sliding window method)

zt=t4+5t3+11t2+11t+4

Standard method:

zt=-∞∞x(τ)y(t-τ)dτ=-∞∞y(τ)x(t-τ)dτ

Q2.

Convoluted signal limits are,

Sum of lower limits <t<Sum of Upper Limits

-2+(-3) <t<2+3

-5<t<5

Q. A linear System with input x(n) and output y(n) related as

yn=k=-∞∞x(k)g(n-2k), where gn=un-u(n-4). Find y(n) when xn=δ(n-2)

Solution:

y(n) when xn=δ(n-2) -> δ(n-k) , here k=2

δn=1 at n=0

δn-2=1 at n=2

yn=x(2)g(n-4)

yn=δ(2-2)g(n-4)= δ(0)g(n-4)= g(n-4)

yn= un-4-u(n-8)

Q. A linear System with input x(n) and output y(n) related as

yn=k=-∞∞x(k)g(n-2k), where gn=un-u(n-4). Find y(n) when xn=u(n)

Solution:

yn=k=-∞∞x(k)g(n-2k)=k=0∞g(n-2k)

Discrete Convolution:

Q. x(n) = {1, 2, 3, 4}, h(n)={1,1,-1,1}

Find z(n)=x(n)* h(n)

If Length of x(n) is m and length of h(n) is n, then total length is m+n-1.

Solution:

|  | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 3 | 4 |
| 1 | 1 | 2 | 3 | 4 |
| -1 | -1 | -2 | -3 | -4 |
| 1 | 1 | 2 | 3 | 4 |

z(n)=x(n)* h(n) = {1, 3, 4, 6, 3, -1,4}

Q. x(n) = {1, 2, 3}, h(n)={1,-2}

|  | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 1 | 1 | 2 | 3 |
| -2 | -2 | -4 | -6 |

z(n)=x(n)* h(n) = {1, 0, -1,-6}

Q. Find the overall response of the given LTI System

Solution:

yt=[xt+xt*h1t+xt*h2t]*h3(t)

Q.

- Find the overall System response

- Find y(n), if x(n) = {1, 2, 3, 4}, h(n)={1,1,-1,1}, where h1(n)=h(n-1), h2(n)=h(n), h3(n)=2h(n) and h3(n)= h(n+1)

Solution:

- Overall response : h1n+h2[n]*h3n*h4[n]= [{1, 2, 3, 4}+{1,1,-1,1}]*{2,2,-2,2}*{1,1,-1,1}={1,2,1,4,4}*{2,2,-2,2}*{1,1,-1,1}

- yn=xn*Overall response

Representation of signal in terms of unit step and unit ramp function.

Solution:

xt=rt+2-2rt+1+rt-1+2ut-3-ut-4

Q.

xt=+1 ut++1ut-a+-3ut-2a+u(t-3a)++1ut-3a+2a-0rt-3a+-2a-2art-4a+(0-(-2a))r(t-5a)

xt=ut+ut-a-3 ut-2a+u(t-3a)+ut-3a+2art-3a-4art-4a+2a r(t-5a)

Q.

- xt=82rt+2-44r(t)

Q. Show that the following signals are orthogonal.

Solution:

t1t2xtytdt=0

04x(t)y(t)dt=01(-1×-1)dt+12(-1×1)dt+23(1×1)dt+34(1×-1)dt

=011 dt+12-1 dt+231 dt+34-1   dt=

=1-1+1-1=0

The signals are orthogonal.

Q. Show that the following signals are orthogonal over the interval [0, 1].

x1t=2, x2t=3(1-2t)

Solution:

t1t2x1tx2dt=0

=>0123(1-2t) dt=01223dt- 01423 t dt

=>223011 dt-42301t dt

=223   t10-423   t2210   =2231-0-223[1-0]=0

The signals are orthogonal.

EXAMPLES

Memoryless

- y(t)=4∗x(t)y(t)

- y[n]=x2[n]y[n]

Non-memoryless Systems

- yn=-∞nxn

- yn=x[n-k] for k≠0

EXAMPLES

Invertible Systems

- y(t)=4x(t)

  - x(t)=14y(t) is the inverse

Non-invertible Systems

- y(t)=x2(t)

cannot determine sign of input from knowledge of output so the system in not invertible

Find if yn=xn+1x(n-1) is causal or non-causal.

Solution: If n=1,

y1=x1+1x(1-1)=x1+1x(0)

System is dependent upon present and past input so it’s causal.

Ex2: yn=2xn+1x2(n) is causal

Ex3: yn=xn+12x(n+1) is non causal

Ex4: yt=xt+xt-1+1x(t+1) is non causal

All non-causal systems are dynamic systems, but all dynamic systems are noncausal.

All static systems are causal but all causal systems are not static.

yt=2x(t)

Solution:

xt=12yt=122xt=x(t), invertible

Ex2. yt=x(2t)

Solution:

xt=y12t=y12.2t=x(t) invertible

Ex2. yt=x2(t) is non-invertible

Q. Ex: yn=x[n], yn=x2[n] are bounded systems

Q. Ex:  yn=1x[n] is unstable system.

Q. yn=nx2(n)

Answer:

Linearity : Ta1x1t+a2x2t=a1y1t+a2y2(t)

y1t=Tx1t=nx12(t)

y2t=Tx2t=nx22(t)

Ta1x1t+a2x2t=n(a1x1t+a2x2t)2

Superpostion theorem fails so its Nonlinear.

Ta1x1t+a2x2t≠a1y1t+a2y2(t)

Q. yn=nx(n)

Answer:

Linear time variant system.

Q. yn=x(n-2)

Answer:

Linear time invariant signal.

yn=x(n)

y0=x(0), It is a static signal

Ex. yt=2x2(t)

y-2=2x2(-2), It is a static signal

Ex. yt=2x(t2), Dynamic

Ex. yt=xn+x(n-1), Dynamic

Ex. t=xn+x(n+2) , Dynamic

Q. Find a system yt=x2(t) is linear or non-linear.

Ans:

Tx1t= x12(t)

Tx2t= x22(t)

Ta1x1t+a2x2t= a1x1t+a2x2t2= (a1x1t)2+(a2x2t)2+2a1x1ta2x2t

a1x1t+a2x2t≠a1Tx1+a2t[x2], so its violating superposition principle. So it’s a Non-Linear system.

Q. Find a system yt=x(t) is linear or non-linear.

Ans:

Tx1t= x1(t)

Tx2t= x2(t)

Ta1x1t+a2x2t=a1x1t+a2x2t

Ta1x1t+a2x2t=a1Tx1+a2t[x2], so it satisfies superposition principle. So it’s a Linear system.